LAN-G1008R

Layer3 GPON OLT

# Highlights

• Compact design, meets various application scenarios

Supports the deployment in diverse scenarios including low density areas, remote areas, sparsely-populated

areas and industry parks.

Supports FTTM and sharing site/rack with wireless base stations.

• Small size and lightweight, easier to delivery and install

Small size and lightweight, easy to transport and install. Supports multiple installation modes, like limited room space,basement, low-voltage room and small rack or   cabinet.

The LAN-G1008R is a pizza-box GPON OLT with

8 GPON ports that meets flexible and fast FTTx access, suiting  the scenarios such as sparse/  remote/cost-sensitive area,smart industrial

park, commercial building and  FTTM, etc.

• Carrier-class security protection, support dynamic

routing protocol RIP&OSPF, ensure the safe operation of network

Supports uplink redundancy protection including LACP STP,RSTP and MSTP, and support RIP&OSPF protocol.  Supports link

protection.

• Lower TCO

Dramatically saves on investment fees in trunk fibers, pipe

engineering, and facilities.  Effectively reduce CapEx and OpEx.

Technical Specifications

| Item |  | LAN-G1008R |
| --- | --- | --- |
| Chassis | Rack | 1U 19Inch Standard Box |
| Uplink Port | QTY | 8 |
|  | RJ45(GE) | 4 |
|  | SFP(GE)/SFP+( 10GE) | 4 |
| GPON Port | QTY | 8 |
|  | Physical Interface | SFP Slots |
|  | Max splitting ratio | 1:128 |
| Management Ports |  | 1* 10/100/1000BASE-T out-band port, 1*CONSOLE port, 1*USB2.0, 1*Type-C USB (for console) |
| Backplane Bandwidth (Gbps) |  | 120 |
| Port Forwarding Rate(Mpps) |  | 124.992 |
| PON Port Specification | Transmission Distance | 20KM |
|  | GPON port speed | Upstream 1.244Gbps, Downstream 2.488Gbps |
|  | Wavelength | TX 1490nm, RX 1310nm |
|  | Connector | SC/UPC |
|  | Fiber Type | 9/125 μm SMF |
|  | Supported PON module level | Class B+, C, C+, C++, C+++,C++++ |
| Power Supply | AC | Working Voltage Range: 100~240V, Rated Voltage: 110V/220V |
|  | DC | Working Voltage Range: -36~-75V, Rated Voltage: -48V/-60V |
| Management Mode |  | EMS, WEB, SNMP, Telnet , CLI |

Management Function

•  SNMP,Telnet,CLI,WEB,SSH v2;

•  Fan Group Control

•  Port Status monitoring and  configuration management

•  Online ONTconfiguration and  management

•  User management

•  Alarm management

Layer2 Switch

•  32K Mac address

•  Support 4096 VLANs

•  Support port VLAN

•  Support VLAN tag/Un-tag, VLAN transparent transmission

•  Support VLAN translation and QinQ

•  Support storm control based on port

•  Support port isolation

•  Support port rate limitation

•  Support 802.1D and 802.1W

•  Support static LACP,Dynamic LACP

•  QoS based on port, VID, TOS and MAC

address

•  Access control list

•   IEEE802.x flowcontrol

•  Port stability statistic and  monitoring

# Multi cast

•   IGMP snooping

•   16K IP Multicast Groups;

DHCP

•  DHCP server,DHCP relay,DHCP snooping

•  DHCP option82

Layer 3 Route

•  ARP proxy

•   12K hardware Host Routes,12K hardware

Subnet Routes

•  Support Radius,Tacacs+

•  Support IP source guard

•  Support static route ,dynamic route RIP

v1/v2,RIPng and OSPF v2/v3;

IPv6

•  Support NDP;

•  Support IPv6 Ping,IPv6 Telnet,IPv6 routing;

•  Support ACL based on source IPv6 address,

destination IPv6 address, L4 port, protocol

type, etc;

•  Support MLD v1/v2 snooping(Multicast

Listener Discovery snooping)

GPON  Function

•  Tcont DBA

•  Gemport traffic

•   In compliant with ITU-T984.x standard

•  Up to 20KM transmission Distance

•  Support data encryption, multi-cast, port VLAN, separation, RSTP, etc

•  Support ONT auto-discovery/link

detection/remote upgrade of software

•  Support VLAN division and user

separation to avoid broadcast storm

•  Support power-off alarm function, easy

for link problem detection

•  Support broadcasting storm resistance function

•  Support port isolation between  different ports

•  Support ACL and SNMP to configure

data packet filter flexibly

•  Specialized design for system

breakdown prevention to maintain

stable system

•  Support RSTP

Dimension (L*W*H)

•  442mm*319mm*43.6mm

Weight

•  Net weight of single power: 4.14kg

•  Gross weight of single power: NAkg Power Consumption

•  40W

Working Temperature

•  -20。C ~+60。C

Storage Temperature

•   40~+70。C

Relative Humidity

•  5~95% (non-condensing)

Ordering Information:

| Product Name | Product Description | Power Configuration | Accessories |
| --- | --- | --- | --- |
| LAN-G1008R | 8*GPON,4*RJ45+4*SFP/SFP+ | 1*AC power 2*AC power 2*DC power 1*AC power + 1* DC power | GPON SFP C++++ module 1G SFP module 10G SFP+ module |