10 Ports Non-open Pre-connectorized Fiber Access

FastConnect Terminal Box

Product Overview

Fiber access terminal box (FAT) also named as optical termination box (OTB), is a compact fiber management box used for FTTH application. Fiber termination box (FTB) is widely used in FTTx cabling for both fiber cabling and cable management.

Application

- FTTH;

- FTTA;

- Trunk Cable Connecting.

Feature

- High compatibility support harden adapter of OptiTap, Slim and FastConnect;

- Strong enough: working under 1000N pulling force long term;

- Installation on Wall/Pole/Aerial mounted, underground;;

- Available with PLC fiber divide;

- Decreased angle surface and height make sure no connector interfering when operating;

- Cost effective: save 40% operating time and less manpower.

Technical Parameter

| Model | Total Value (dB) | 1x2 FBT High Power (dB) | 1x2 FBT + 1x8 PLC (dB) |
| --- | --- | --- | --- |
| 90/10 | ≤21.54 | ≤ 0.73 | ≤ (11.04+10.5) |
| 85/15 | ≤ 20.78 | ≤ 1.13 | ≤ (10.28+10.5) |
| 80/20 | ≤ 18.25 | ≤ 1.25 | ≤ (7.75+10.5) |
| 70/30 | ≤ 16.51 | ≤ 2.22 | ≤ (6.01+10.5) |
| 60/40 | ≤ 15.32 | ≤ 2.73 | ≤ (4.82+10.5) |
| 1:8 | ≤ 10.50 | ≤ N/A | ≤ 10.5 |

Specification Details

| Dimension (L x W x H) | 198 x 160 x 70mm |
| --- | --- |
| Waterproof Level | IP68 |
| Port Type Solution | 10pcs of Harden FastConnect Adapters |
| Color | Black |
| Material | PC + ABS |
| Max Capacity | 10 Ports |
| UV resistance | ISO 4892-3 |
| Fire protection rating | UL94-V0 |
| Number of PLC (Solution) | 1x2 FBT Uneven PLC + 1x8 PLC |
| Warranty Life Time (Non-artificial damage) | 5 Years |

Mechanical Parameter

| Atmospheric Pressure | 70KPa~106Kpa |
| --- | --- |
| Lid opening angle for operating | No, 100% sealed off (Ultrasonic crimping) |
| Tensile Resistance | ＞1000N |
| Crush Resistance | ＞2000N/10cm2 Pressure, time 1min |
| Insulation resistance | ＞2×104MΩ |
| Compressive Strength | 15KV(DC)/1min no breakdown and no arcing. |
| Relative Humidity | ≤93％ (+40℃) |

| Storage Temperature | -40℃ ～ +85℃ |
| --- | --- |
| Operating Temperature | -40℃ ～ +60℃ |
| Installation Temperature | -40℃ ～ +60℃ |

Application Scheme

➋ 3 PCS Uneven Box

➊ Fiber Cable

➌ End Box

➋ Drop Flat Cable

1x9

1 FO

1x9

1 FO

1x9

1 FO

1x8

50m

50m

50m

1 FO

➊ Hub Box

1 FO

1 FO

CO

MDU single fiber solution

Indoor Duct

Outdoor multi-fiber solutions

Ground

Outdoor single fiber solution

Ground / Duct

PLC Splitter Diagram

Drawing of PLC Splitter: 1 x 9 Solution

35mm

1:8 PLC Splitter

15cm

1: 2 FBT, 60/40, 70/30, 80/20

85/15, 90/10 Optional	INPUT

S/N: XXXXXXXXX

50cm

50cm

SC/APC Connector (Green Color)

900um Loose Tube 8 Colors

SC/APC Connector (Green Color)

Port 1

Port 2

Side B

Port 3

Side A

Port 4

Port 5

Port 6

Port 7

Port 8

249

290

240

59

97

300

Including Accessories

| Item Description | Picture | Qty |
| --- | --- | --- |
| L40mm Screw |  | 4 |
| Ø15mm*6mm Hex Socket Head Cap Screw |  | 4 |
| Ø20mm*6mm Hex Socket Head Cap Screw |  | 4 |
| Ø8mm*3.5mm Hex Nuts |  | 4 |
| Ø8mm*40mm Drilling drywall anchors |  | 4 |
| Ø9mm Metal Lock Washer |  | 4 |
| Ø260mm Stainless Steel Hose Ring |  | 2 |
| Ø3mm*12cm L Handle Allen Wrench Kit |  | 1 |
| Wing Nuts 304 Stainless |  | 4 |
| Cable Storage Kits |  | 2 |
| Aerial-mounted Kit |  | 2 |

Product Inner Box Packing

|  | Size (cm) | 25cm*25cm*13cm |
| --- | --- | --- |
|  | QTY (pcs/box) | 1 |
|  | Net Weight (kg/pcs) | 0.61 |

Accessories Inner Box Packing

|  | Size (cm) | 25*25**15cm |
| --- | --- | --- |
|  | Set | 8pcs/set |
|  | G.W (kg/box) | 0.6 |

Master Carton for Product

|  | Size (cm) | 63cm*50cm*48cm |
| --- | --- | --- |
|  | QTY (pcs/box) | 12 |
|  | Net Weight (kg/Carton) | 7.2 |

Master Carton for Accessories

|  | Size (cm) | 63cm*50cm*48cm |
| --- | --- | --- |
|  | QTY (Sets/box) | 12 |
|  | Net Weight (kg/Carton) | 7.5 |