Fast Connector

product description:

1. Push-tube type fastening structure, optical fiber fastening, stable and reliable docking.

2. One-piece structure, simple structure, stable and reliable, excellent comprehensive cost.

3.Simple operation, fast construction speed, high installation success rate, long life cycle, simple and convenient post-maintenance.

Product Structure：

Measured the Structure

Product length：

50mm ≤L≤60mm(≈54m)

Ptical Characteristics：

| Category | Specification |
| --- | --- |
| Aoolicable for | Drop cable. Indoor cable |
| Fiber mod | Single mode or Multi mode |
| Fiber type | G.652D, G.657A2, |
| Fiber cable | 3.0*2.0mm rubber-insulated optical cable |
| Fiber diameter | 0.125mm |
| Force holds 2 x 3mm cable type at the beginning the fast connector | ≥ 50 N, loss ≤ 0.3 dB |
| Fiber lock | Lock holds fiber reliability, bears hang force ≥ 1 N, attenuation changes ≤ 0.2 dB |
| Working wavelength | 1310 nm, 1490 nm, 1550 nm |

| Operation time | S | About 50(no fiber cut) |
| --- | --- | --- |
| Insertion Loss | dB | ≤0.5 |
| Insertion Loss Typical value | dB | 0.3 |
| Return Loss | dB | UPC≥50, APC≥55 |
| Fastening strength of naked fiber | N | ≥3 |
| Tensile strength | N | ≥45 |
| Reusable | Time | ≥10 time |
| Operating Temperature |  | -40~+85 |
| On-line tensile strength（20N） | dB | ΔIL≤0.3 |
| Mechanical durability (500 times) | dB | ΔIL≤0.3 |
| Drop-off test (drop-off height4m, once per direction, totally 3 times) | dB | ΔIL≤0.3 |

Environmental performance requirements:

| Category | Specification |  |  |  |
| --- | --- | --- | --- | --- |
|  | Condition | IL Variation | RL Variation | Appearance |
| High TEMP | 85℃, 168H | ≤0.3 | ≤5 | No mechanical da mage, like deform ation, cracking and untight |
| Low TEMP | -40℃, 168H | ≤0.3 | ≤5 |  |
| Temperature Cycle | Cycle: 8 hours Total: 12 cycles / 168 hours Temperature Maintenance: 1 hour | ≤0.3 | ≤5 |  |
| Temperature/ Humidity | 85℃, 95%RH, 168H | ≤0.3 | ≤5 |  |
| Immersion | 25℃, 168H | ≤0.3 | ≤5 |  |
| Salt Mis | 5% Nacl, 6.5＜PH＜ 7.5,35℃,168H | ≤0.3 | ≤5 |  |

Mechanical Performance Requirement

| Category | Specification |  |  |  |
| --- | --- | --- | --- | --- |
|  | Condition | IL Variation | RL Variation | Appearance |
| Impact | Height: 4m; 1 drop each axis; 3 cycles | ≤0.3 | ≤5 | No mechanical da mage, like deforma tion, cracking and untight |
| Vibration | Frequency: 10 ～ 50HZ; Sweep: 45 times / min; amplitude: 0.75mm; Duration:30min each axial | ≤0.3 | ≤5 |  |
| Torsional | Pull：15N; The distance between Loading location and fast connector: L = 22-28cm; Rate：10times / min. Times: | ≤0.3 | ≤5 |  |

| Tensile | Pull：50N; The distance between Loading lo cation and fast connector: L = 3M; Duration:1min | ≤0.3 | ≤5 |  |
| --- | --- | --- | --- | --- |

Product Packaging Information：

Blister Box Packing

Adapter (supports strip and cut fiber)：

1 adapter / 50 fast con nectors          1 Operation Manual/ 100 fast con ne